PROJECT REPORT

AI-Driven Digital Twin for Smart Factory Operations

| Submitted by | Akshat Kaushal |
| --- | --- |
| Programme | B.Tech – Electronics & Communication Engineering |
| Institution | ABES Engineering College, Ghaziabad |
| Academic session | 2026–27 |
| Project domain | Embedded Systems & IoT |
| Implementation status | Simulated telemetry generated by an ESP32 and sent over MQTT; no physical sensors connected |

# Abstract

This project presents an educational prototype of an AI-assisted digital-twin monitoring pipeline for a smart-factory machine. Simulated machine telemetry is received through a message-ingestion service, stored in InfluxDB Cloud, analysed by a PyTorch neural-network classifier and visualized in Grafana. The current software accepts seven parameters: temperature, pressure, motor speed, vibration, current, voltage and humidity. The prototype demonstrates the software flow from telemetry ingestion to status visualization. The supplied AI training set is very small and has not been independently validated; therefore, the output is treated as an experimental classification score rather than an industrial failure probability . The ESP32 publishes simulated telemetry over MQTT; connecting physical sensors (for example a BME680) is planned future work pending hardware verification.

Smart manufacturing uses connected devices and software systems to make machine information easier to monitor. A digital-twin concept maintains a digital representation of an asset using data associated with that asset. This project focuses on a small-scale monitoring prototype rather than a production-grade digital twin.

# Chapter 2 – Problem and objectives

The project aims to demonstrate how machine telemetry can be collected, stored, classified and visualized. The objectives are to receive seven numeric parameters, store time-series records, run a prototype classifier, show status and trends, and document verification.

# Chapter 3 – System design

The current software architecture comprises an ESP32 publishing simulated telemetry, an EMQX Cloud MQTT broke, Flask receiver, InfluxDB Cloud, PyTorch AI worker and Grafana Cloud. The Flask endpoint maps incoming topic names to fields and writes them to factory_esp32. The AI worker queries the latest complete field values and writes its output to factory_ai.

# Chapter 4 – Implementation

The supplied cloud_receiver/app.py implements the HTTP health endpoint and /mqtt ingestion route. It validates JSON presence, recognized topic names and numeric payloads. cloud_receiver/ai_worker.py defines a feed-forward neural network with seven inputs and trains it on eight hard-coded examples. It queries InfluxDB, checks that all seven fields are present and that the latest timestamp is not older than 15 seconds, then assigns NORMAL, WARNING or FAULT using 0.50 and 0.80 thresholds. src/esp32_ai.py contains a second local AI/InfluxDB processing implementation; it should not be run concurrently with the cloud AI worker unless duplicate writes are intentionally managed.

# Chapter 5 – Results and testing

No test execution evidence was supplied with the repository ZIP. Therefore, this report does not claim measured latency, accuracy, recall, uptime, successful physical sensor acquisition or passing test cases. The accompanying Test_Cases.xlsx lists the tests and starts each test in Not Run status. Results must be filled after execution.

# Chapter 6 – Limitations

The current telemetry source for demonstration is simulated.

The firmware contains no physical-sensor code; it generates simulated values (see firmware/esp32_machine_simulator in the repository.

The classifier is trained on only eight examples and lacks an independent test set.

No feature normalization or probability calibration is shown in the supplied AI code.

The receiver does not currently check InfluxDB HTTP error status before returning success.

The current receiver maps seven fields; power is not a supported telemetry field in this implementation.

# Chapter 7 – Future work

Integrate a BME680 (temperature, humidity, pressure) and other physical sensors, and verify pins, units and calibration.

Create a repeatable simulator with labelled normal and fault scenarios.

Expand the training dataset, scale features, split train/validation/test data and report metrics.

Persist a trained model artifact and version its preprocessing/model parameters.

Correct receiver error handling and add structured logging.

Verify Grafana queries, alerts and dashboard screenshots.

Add multi-machine support and additional digital-twin features only after core telemetry is stable.

# Conclusion

The project provides a foundation for an Industrial IoT monitoring prototype with cloud time-series storage, AI-assisted classification and dashboard visualization. The current deliverable should be evaluated as a simulated software prototype. Hardware integration and reliable AI validation are important next steps before describing it as a validated physical predictive-maintenance system.

# References / project artefacts

HCLTech, Project Creation Guide – Embedded Systems & IoT, Version 1.0, September 2026 (user-provided guide).

Project source repository ZIP supplied by the project owner (README.md, cloud_receiver/app.py, cloud_receiver/ai_worker.py, src/esp32_ai.py, grafana/queries.md, requirements files and docs/SRS.docx).