SOFTWARE REQUIREMENTS SPECIFICATION

(SRS Document)

for

AI-Driven Digital Twin for Smart Factory Operations

Version 1.0

Submitted by

Akshat Kaushal

Roll No.: 2400320310019

B.Tech – Electronics and Communication Engineering (ECE-A)

3rd Year

ABES Engineering College, Ghaziabad

Academic Session: 2026–27

Document Control

| Field | Details |
| --- | --- |
| Project Title | AI-Driven Digital Twin for Smart Factory Operations |
| Document Type | Software Requirements Specification (IEEE 830 style) |
| Author | Akshat Kaushal |
| Roll Number | 2400320310019 |
| Branch / Section | B.Tech ECE – A (3rd Year) |
| Institution | ABES Engineering College |
| Version | 1.0 |
| Date | 30 September 2026 |
| Status | Draft for review |

Revision History

| Version | Date | Author | Description |
| --- | --- | --- | --- |
| 0.1 | 22 Sep 2026 | Akshat Kaushal | Initial outline and scope definition |
| 1.0 | 30 Sep 2026 | Akshat Kaushal | Complete SRS draft |

Table of Contents

1. Introduction

2. Overall Description

3. System Architecture Overview

4. Functional Requirements

5. External Interface Requirements

6. Non-Functional Requirements

7. Data Requirements

8. Use Cases

9. Hardware and Software Requirements

10. Risk Analysis

11. Testing and Acceptance Criteria

12. Future Scope

Appendix A: Requirements Traceability Matrix

Appendix B: Glossary

# 1. Introduction

## 1.1 Purpose

This Software Requirements Specification (SRS) describes the functional and non-functional requirements of the project “AI-Driven Digital Twin for Smart Factory Operations”. It is intended to serve as the baseline agreement between the developer, the project guide, and evaluators on what the system shall do, and as the reference for design, implementation, and testing.

## 1.2 Scope

The proposed system creates a virtual replica (digital twin) of a small-scale smart factory production line. Sensor data from machines, such as temperature, vibration, current, and conveyor speed, is acquired by embedded nodes (an ESP32, which generates simulated telemetry in the current prototype), transmitted to a cloud/edge platform, and used to continuously update the twin. AI/ML models running on the twin perform anomaly detection, predictive maintenance, and what-if simulation, and results are presented through a real-time dashboard with alerts. In the current prototype no physical sensors are connected, and the requirements marked Future work in this document are not implemented.

The product will:

- Acquire and pre-process real-time machine and environment data.

- Maintain a synchronized virtual model of each machine and the overall production line.

- Detect abnormal behaviour and predict failures before they occur.

- Provide visualization, alerts, historical analytics, and scenario simulation.

The product will not directly control safety-critical actuators in its initial version, and it is not certified for deployment in a live industrial plant. It is a prototype for academic and demonstration purposes.

## 1.3 Definitions, Acronyms and Abbreviations

| Term | Meaning |
| --- | --- |
| Digital Twin (DT) | A virtual model that mirrors the state and behaviour of a physical asset in near real time |
| IoT | Internet of Things |
| IIoT | Industrial Internet of Things |
| MQTT | Message Queuing Telemetry Transport, a lightweight publish/subscribe protocol |
| ML / AI | Machine Learning / Artificial Intelligence |
| PdM | Predictive Maintenance |
| RUL | Remaining Useful Life |
| UART / I2C / SPI | Serial communication interfaces used between embedded devices and sensors |
| SRS | Software Requirements Specification |
| API | Application Programming Interface |
| KPI | Key Performance Indicator (e.g., OEE, downtime) |
| OEE | Overall Equipment Effectiveness |

## 1.4 References

- IEEE Std 830-1998, Recommended Practice for Software Requirements Specifications.

- ISO 23247, Digital Twin Framework for Manufacturing.

- MQTT Version 3.1.1 Specification, OASIS Standard.

- ESP32 and Arduino official documentation.

- PyTorch, Flask, InfluxDB, Grafana and EMQX documentation.

## 1.5 Document Overview

Section 2 gives the overall product description. Section 3 outlines the architecture. Sections 4 to 7 detail functional, interface, non-functional, and data requirements. Sections 8 to 12 cover use cases, environment, risks, testing, and future scope. Appendices contain the traceability matrix and glossary.

# 2. Overall Description

## 2.1 Product Perspective

Traditional factories rely on periodic inspection and reactive maintenance, which causes unplanned downtime and quality loss. This project is a self-contained system that fits into the Industry 4.0 paradigm. It combines an embedded node (ESP32, ECE domain), communication networks, cloud services, and AI analytics. In this version the physical process is simulated, so the loop with a real machine is not yet closed.

## 2.2 Product Functions (Summary)

- Sensor data acquisition and edge pre-processing.

- Secure data transmission to the cloud/edge server.

- Digital twin model creation and state synchronization.

- Anomaly detection and predictive maintenance using ML.

- What-if simulation of operating conditions.

- Real-time dashboard, alerting, and report generation.

- User authentication and role-based access.

## 2.3 User Classes and Characteristics

| User Class | Description | Technical Level |
| --- | --- | --- |
| Plant Operator | Monitors live status and responds to alerts | Basic |
| Maintenance Engineer | Views health scores, failure predictions, and history | Intermediate |
| Plant Manager | Reviews KPIs, downtime, and efficiency reports | Basic |
| System Administrator | Manages users, devices, and model configuration | Advanced |
| Developer / Evaluator | Tests, simulates faults, and validates the system | Advanced |

## 2.4 Operating Environment

- Edge layer: an ESP32 node that generates simulated telemetry in this version (no physical sensors connected).

- Communication: Wi-Fi with MQTT over TLS and HTTP.

- Server/cloud: Python (Flask) receiver, EMQX Cloud (MQTT broker), InfluxDB Cloud and Grafana Cloud.

- Client: modern web browser on Windows, Linux, or Android.

## 2.5 Design and Implementation Constraints

- The prototype must run on low-cost hardware and free or student-tier cloud services.

- Hardware behaviour may be simulated in the ESP32 firmware where physical equipment is unavailable.

- Internet bandwidth and latency limit the achievable update rate.

- The project must be completed within the academic semester timeline.

## 2.6 Assumptions and Dependencies

- Sensor readings are available at a fixed sampling interval (typically 1 to 5 seconds).

- A stable Wi-Fi or network connection is available.

- Historical or synthetic fault data is available to train and validate ML models.

- Third-party services (EMQX Cloud, InfluxDB Cloud, Grafana Cloud) remain accessible.

# 3. System Architecture Overview

The system follows a layered architecture, from the physical process at the bottom to the user interface at the top.

| Layer | Components | Responsibility |
| --- | --- | --- |
| 1. Physical Layer | Motors, conveyor, machines, process variables (simulated in this version) | Source of real-world behaviour |
| 2. Sensing & Edge Layer | ESP32 node generating simulated telemetry (no physical sensors) | Generate and publish telemetry |
| 3. Communication Layer | Wi-Fi, MQTT over TLS (EMQX Cloud), HTTP | Reliable transport of telemetry and commands |
| 4. Data & Storage Layer | InfluxDB Cloud time-series database | Store raw, processed, and event data |
| 5. Digital Twin & AI Layer | PyTorch anomaly classifier (prototype); other components are future work | Mirror physical state and generate insights |
| 6. Application Layer | Grafana Cloud dashboard (alerts, reports and REST API are future work) | Visualization, notification, and user interaction |

Data flow: Simulated telemetry (ESP32) → MQTT (EMQX Cloud) → Flask receiver → InfluxDB Cloud → AI worker → Grafana dashboard. Feedback to the device (operator commands or set-points) is future work.

# 4. Functional Requirements

Priority levels: H = High (must have), M = Medium (should have), L = Low (nice to have).

Implementation status: each requirement in Sections 4 and 6 ends with a status label. [Implemented] means it works in the current prototype, [Partial] means it is only partly implemented or has not been measured, and [Future work] means it is a target for a later version and is not part of the demonstrated prototype. Use cases in Section 8 that depend on Future work requirements are also future work. All telemetry is simulated by the ESP32 firmware; no physical sensors are connected.

## 4.1 Data Acquisition (Edge Module)

| ID | Requirement | Description | Pri. |
| --- | --- | --- | --- |
| SRS-01 | Sensor reading | The system shall read temperature, vibration, current/power, and speed values from each machine node at a configurable interval. [Partial: seven simulated values are generated by the ESP32 every 3 s; no physical sensors; interval is fixed in firmware] | H |
| SRS-02 | Signal conditioning | The edge node shall filter noise (e.g., moving average) and validate readings against physical limits. [Future work] | M |
| SRS-03 | Time-stamping | Each reading shall carry a timestamp and a unique device and machine ID. [Partial: machine tag machine1; timestamp is assigned when the data is written to InfluxDB] | H |
| SRS-04 | Local buffering | The node shall buffer data locally when the network is down and upload it when connectivity returns. [Future work] | M |

## 4.2 Communication and Data Ingestion

| ID | Requirement | Description | Pri. |
| --- | --- | --- | --- |
| SRS-05 | Telemetry publishing | Nodes shall publish data to the broker/cloud using MQTT or HTTP in a defined JSON format. [Implemented: MQTT over TLS, one topic per parameter with a numeric payload; the broker forwards topic and payload to the receiver as JSON] | H |
| SRS-06 | Gateway support | The system shall support a UART-based sensor node to gateway node link before forwarding to the cloud. [Future work] | M |
| SRS-07 | Data validation | The ingestion service shall reject malformed or out-of-range packets and log the error. [Partial: malformed or non-numeric packets are rejected; no physical range check] | H |
| SRS-08 | Command channel | The system shall allow authorized users to send configuration commands (e.g., sampling rate) to nodes. [Future work] | L |

## 4.3 Digital Twin Management

| ID | Requirement | Description | Pri. |
| --- | --- | --- | --- |
| SRS-09 | Twin creation | The system shall create a digital twin object for every registered machine, storing its properties and current state. [Future work: machine1 is represented only by its stored time-series data] | H |
| SRS-10 | State synchronization | The twin state shall be updated from incoming telemetry within 5 seconds of receipt. [Partial: not measured] | H |
| SRS-11 | Production line model | The system shall model the production line as connected machines and show material flow and dependencies. [Future work: one simulated machine only] | M |
| SRS-12 | Health index | The system shall compute a health score (0 to 100) for each machine from its sensor data. [Future work: the prototype outputs a model score and NORMAL/WARNING/FAULT status instead] | H |
| SRS-13 | Twin history | The system shall preserve historical twin states for replay and analysis. [Partial: telemetry and AI output are retained in InfluxDB; no replay feature] | M |

## 4.4 AI and Analytics

| ID | Requirement | Description | Pri. |
| --- | --- | --- | --- |
| SRS-14 | Anomaly detection | The system shall detect abnormal behaviour in real time using an ML model (e.g., Isolation Forest or Autoencoder). [Partial: small PyTorch classifier trained on a very small hard-coded dataset; not independently validated or calibrated] | H |
| SRS-15 | Predictive maintenance | The system shall predict probable failures or estimate RUL using a trained model (e.g., Random Forest or LSTM). [Future work] | H |
| SRS-16 | What-if simulation | The user shall be able to change parameters (load, speed, ambient temperature) and view simulated twin response without affecting the physical system. [Future work] | M |
| SRS-17 | Model training | The system shall support retraining/updating models from stored historical data. [Future work: the current model is trained from hard-coded examples at start-up] | M |
| SRS-18 | KPI computation | The system shall compute KPIs such as uptime, downtime, throughput, and OEE. [Future work] | M |

## 4.5 Visualization and Alerts

| ID | Requirement | Description | Pri. |
| --- | --- | --- | --- |
| SRS-19 | Live dashboard | The dashboard shall show real-time values, trends, and machine status (Normal / Warning / Critical). [Implemented in Grafana: real-time values, trends and NORMAL / WARNING / FAULT status] | H |
| SRS-20 | Twin view | The dashboard shall provide a graphical or schematic view of the factory line reflecting the live twin state. [Future work] | M |
| SRS-21 | Alerts | The system shall raise alerts on the dashboard and by email/SMS/notification when thresholds are breached or anomalies are detected. [Future work] | H |
| SRS-22 | Historical charts | Users shall be able to view charts for a selected machine and time range. [Partial: available through the Grafana time-range selector] | M |
| SRS-23 | Reports | The system shall export reports (PDF/CSV) of events, KPIs, and maintenance recommendations. [Future work] | L |

## 4.6 User and System Administration

| ID | Requirement | Description | Pri. |
| --- | --- | --- | --- |
| SRS-24 | Authentication | Users shall log in with a username and password before accessing the system. [Future work: access relies on the Grafana and InfluxDB accounts only] | H |
| SRS-25 | Role-based access | The system shall restrict functions according to user roles (Operator, Engineer, Manager, Admin). [Future work] | M |
| SRS-26 | Device registration | The administrator shall add, edit, and remove machines and devices. [Future work] | M |
| SRS-27 | Logging | The system shall log data events, alerts, user actions, and errors. [Partial: ESP32 Serial Monitor output only] | M |

# 5. External Interface Requirements

## 5.1 User Interfaces

- A responsive web dashboard accessible from desktop and mobile browsers.

- Colour-coded status indicators (green, amber, red), interactive charts, and a machine detail page.

- Simple forms for login, device registration, and simulation parameters.

## 5.2 Hardware Interfaces

| Interface | Description |
| --- | --- |
| Microcontroller | ESP32 with Wi-Fi (used in this version); Arduino Uno is not used |
| Sensors | None connected in this version. Planned: BME680 (temperature, humidity, pressure); other sensors to be selected |
| Actuators / indicators | None in this version |
| Simulation | Machine behaviour is simulated in the ESP32 firmware |

## 5.3 Software Interfaces

- MQTT broker: EMQX Cloud.

- Cloud services: InfluxDB Cloud (bucket Smart_factory) and Grafana Cloud.

- Python libraries: Flask, requests, influxdb-client, python-dotenv and PyTorch (see requirements.txt); the ESP32 firmware uses the PubSubClient library.

- Visualization: Grafana Cloud.

## 5.4 Communication Interfaces

- Wi-Fi (IEEE 802.11 b/g/n) for node-to-cloud communication.

- MQTT over TCP/IP with TLS (port 8883; the prototype firmware does not verify the broker certificate) and HTTP/HTTPS calls.

- UART gateway link: not used in this version.

- MQTT payloads are plain numeric values, one topic per parameter; the broker forwards topic and payload to the receiver as JSON.

Sample message forwarded to the receiver (example):

{ "topic": "factory/machine1/temperature", "payload": "36.4" }

# 6. Non-Functional Requirements

| ID | Category | Requirement |
| --- | --- | --- |
| NSRS-01 | Performance | End-to-end latency from sensor reading to dashboard update shall not exceed 5 seconds under normal network conditions. [Partial: publish interval is 3 s; end-to-end latency not measured] |
| NSRS-02 | Performance | The system shall support at least 10 machine nodes sending data every 2 seconds without data loss. [Future work: one simulated machine only] |
| NSRS-03 | Accuracy | The anomaly detection model shall achieve at least 90% recall on the test dataset, with a false-alarm rate below 10%. [Future work: the prototype model has not been independently validated or calibrated] |
| NSRS-04 | Reliability | The system shall achieve at least 95% uptime during demonstration and testing periods. [Partial: not measured] |
| NSRS-05 | Availability | Edge nodes shall recover automatically after power or network interruption. [Partial: firmware reconnects to Wi-Fi and MQTT; not tested] |
| NSRS-06 | Security | Credentials shall be stored hashed, communication shall be encrypted where supported, and unauthorized access shall be blocked. [Partial: MQTT uses TLS but the broker certificate is not verified; cloud credentials are kept in environment variables and not committed; user login is future work] |
| NSRS-07 | Usability | A new user shall be able to operate the dashboard after 15 minutes of guidance. [Partial: not evaluated] |
| NSRS-08 | Scalability | The design shall allow additional machines and sensors to be added without changing the core architecture. [Partial: not evaluated] |
| NSRS-09 | Maintainability | Code shall be modular, commented, and version-controlled (Git). [Implemented: source is in a Git/GitHub repository] |
| NSRS-10 | Portability | The software shall run on Windows and Linux and be accessible from any standard browser. [Partial: not verified on Linux] |
| NSRS-11 | Cost | The prototype shall use low-cost components and free or student-tier services. [Partial: cloud service plans to be confirmed] |
| NSRS-12 | Safety | The prototype shall not issue control commands that can damage equipment or endanger people. [Implemented: the prototype issues no control commands] |

# 7. Data Requirements

## 7.1 Data Entities

| Entity | Key Attributes |
| --- | --- |
| Machine | machine_id, name, type, location, status, install_date |
| Sensor | sensor_id, machine_id, type, unit, range_min, range_max |
| Telemetry | record_id, machine_id, timestamp, temperature, vibration, current, speed |
| TwinState | machine_id, timestamp, health_score, predicted_state, RUL |
| Alert | alert_id, machine_id, timestamp, severity, message, acknowledged |
| User | user_id, name, role, password_hash, email |
| Model | model_id, type, version, trained_on, accuracy |

## 7.2 Data Handling

- Raw telemetry shall be retained for at least 30 days and aggregated data for at least 1 year (storage permitting).

- Training datasets may combine collected data with synthetic or public benchmark data (e.g., NASA bearing / C-MAPSS datasets).

- Data shall be normalized and cleaned (missing values, outliers) before model training.

# 8. Use Cases

| ID | Use Case | Actor | Summary |
| --- | --- | --- | --- |
| UC-01 | Monitor live status | Operator | View real-time machine values and status on the dashboard |
| UC-02 | Receive alert | Operator, Engineer | System raises an alert when an anomaly or threshold breach occurs; the user acknowledges it |
| UC-03 | View failure prediction | Maintenance Engineer | Check predicted failures and RUL, and schedule maintenance |
| UC-04 | Run what-if simulation | Engineer, Developer | Modify parameters in the twin and observe the predicted impact |
| UC-05 | Analyse KPIs and reports | Manager | Review uptime, throughput, and OEE, and export reports |
| UC-06 | Manage devices and users | Administrator | Register machines, configure sensors, and manage user accounts |
| UC-07 | Retrain model | Administrator, Developer | Update ML models with new historical data |

Detailed flow for UC-02 (Receive alert):

- Precondition: the machine node is online and sending telemetry.

- Main flow: (1) the anomaly model flags abnormal data; (2) the system marks the twin state as Warning/Critical; (3) an alert is generated and shown on the dashboard; (4) a notification is sent; (5) the user acknowledges the alert and views details.

- Alternate flow: if the notification service fails, the alert remains on the dashboard and the failure is logged.

- Postcondition: the alert is stored with its acknowledgement status.

# 9. Hardware and Software Requirements

## 9.1 Hardware

| Item | Specification |
| --- | --- |
| Development PC | Intel i5 or equivalent, 8 GB RAM or more, 256 GB SSD |
| Microcontrollers | ESP32 board (Arduino Uno is not used) |
| Sensors and modules | None connected in this version (planned: BME680) |
| Network | Wi-Fi router with internet access |

## 9.2 Software

| Item | Specification |
| --- | --- |
| Operating System | Windows 10/11 or Ubuntu |
| Simulation | Firmware-based telemetry simulation (no physical sensors) |
| Embedded IDE | Arduino IDE |
| Programming | Python 3.10+, C/C++ (Arduino) |
| AI/ML | PyTorch |
| Backend / Cloud | Flask, EMQX Cloud (MQTT), InfluxDB Cloud, Grafana Cloud |
| Version Control | Git and GitHub |

# 10. Risk Analysis

| Risk | Impact | Mitigation |
| --- | --- | --- |
| Real factory data is unavailable | Medium | Use simulated machine behaviour and public benchmark datasets |
| Network or cloud service outage | High | Local buffering at nodes and a local fallback broker |
| ML model gives poor accuracy | High | Feature engineering, model comparison, and more training data |
| Hardware component failure or unavailability | Medium | Proteus simulation as a backup |
| Free-tier cloud limits exceeded | Medium | Reduce sampling rate and aggregate data at the edge |
| Schedule overrun | Medium | Prioritize High requirements and deliver in phases |

# 11. Testing and Acceptance Criteria

Each test case has a unique Test ID (TC-xx) and is mapped to the requirements it verifies. The same mapping is summarised in Appendix A. All test cases are recorded as Not Run until they are executed and evidence is saved; test cases for requirements marked Future work cannot be executed in this version.

| Test ID | Test Type | Test Description | Acceptance Criterion | Verifies |
| --- | --- | --- | --- | --- |
| TC-01 | Unit | Read sensors at the configured interval; check filtering, limits, timestamp and device ID | Valid, time-stamped readings at the set interval | SRS-01, 02, 03 |
| TC-02 | Unit | Disconnect the network, then restore it | Buffered data uploaded with no loss | SRS-04, NFR-05 |
| TC-03 | Integration | Publish valid and malformed packets from node to broker (incl. UART gateway path) | Valid packets accepted; malformed ones rejected and logged | SRS-05, 06, 07 |
| TC-04 | Integration | Send telemetry and observe twin state and history | Twin updates within 5 s; history stored | SRS-09, 10, 11, 13, NFR-01 |
| TC-05 | Unit | Compute health index for reference datasets | Score is within 0 to 100 and matches expected values | SRS-12 |
| TC-06 | Model validation | Evaluate anomaly model on held-out data | Recall ≥ 90%; false alarms < 10% | SRS-14, NFR-03 |
| TC-07 | Model validation | Evaluate predictive maintenance model; retrain with new data | Accuracy meets target; retraining completes successfully | SRS-15, 17 |
| TC-08 | Simulation | Change load, speed and temperature in what-if mode | Twin responds; physical system unaffected | SRS-16 |
| TC-09 | Fault injection | Simulate overheating, overload and vibration spikes | Correct status change and alert for each fault | SRS-19, 21 |
| TC-10 | Functional | Open twin view, historical charts, KPIs and export reports | Views load correctly; KPI values and exports are correct | SRS-18, 20, 22, 23 |
| TC-11 | Security | Try valid and invalid logins; access pages with each role | Unauthorized access denied; roles restricted | SRS-24, 25, NFR-06 |
| TC-12 | Functional | Register a device, send a command and check the logs | Device added; command applied; events logged | SRS-08, 26, 27 |
| TC-13 | Performance | Run 10 nodes at 2-second intervals for 30 minutes | No data loss; latency within limit; uptime ≥ 95% | NFR-02, NFR-04 |
| TC-14 | Usability | Sample users perform UC-01 to UC-05 | Tasks completed without assistance | NFR-07, UC-01 to UC-05 |

For this prototype version, acceptance is limited to the requirements marked Implemented or Partial in Section 4 (SRS-01, 03, 05, 07, 10, 13, 14, 19, 22, 27), demonstrated through their test cases with recorded evidence. Requirements marked Future work are excluded from acceptance of this version.

# 12. Future Scope

- Integration with real PLCs and industrial protocols such as OPC UA and Modbus.

- Closed-loop control where the twin recommends or applies optimal set-points.

- Edge AI using TinyML for on-device anomaly detection.

- 3D visualization of the factory and AR/VR-based monitoring.

- 5G/private network support and blockchain-based data integrity.

- Energy optimization and quality prediction modules.

# Appendix A: Requirements Traceability Matrix

| Objective | Requirements | Test IDs |
| --- | --- | --- |
| Real-time data acquisition | SRS-01 to SRS-04 | TC-01, TC-02 |
| Reliable communication | SRS-05 to SRS-08, NFR-01, NFR-05 | TC-02, TC-03, TC-04, TC-12 |
| Digital twin synchronization | SRS-09 to SRS-13 | TC-04, TC-05 |
| Intelligent analytics | SRS-14 to SRS-18, NFR-03 | TC-06, TC-07, TC-08, TC-10 |
| Visualization and alerting | SRS-19 to SRS-23 | TC-09, TC-10 |
| Security and administration | SRS-24 to SRS-27, NFR-06 | TC-11, TC-12 |
| Performance and usability | NFR-02, NFR-04, NFR-07 | TC-13, TC-14 |

# Appendix B: Glossary

| Term | Definition |
| --- | --- |
| Anomaly | A data pattern that deviates significantly from normal machine behaviour |
| Edge computing | Processing data near its source, on the device, before sending it to the cloud |
| Health index | A numerical score indicating the condition of a machine |
| Latency | Time delay between a physical event and its reflection in the twin |
| Smart Factory | A manufacturing facility using connected devices, data, and AI for autonomous, optimized operation |
| Synchronization | Keeping the digital twin state consistent with the physical asset |